Journal of Power and Energy Engineering, 2015, 3, 31-45

Published Online June 2015 in SciRes. http://www.scirp.org/journal/jpee http://dx.doi.org/10.4236/jpee.2015.36004

Modeling of Criteria Air Pollutant Emissions from Selected Nigeria Petroleum Refineries

Temitayo Oladimeji1, Jacob Sonibare2, Moradeyo Odunfa3, Augustine Ayeni1

1

Chemical Engineering Department, Covenant University, Ota, Nigeria

2

Chemical Engineering Department, Obafemi Awolowo University, Ile-Ife, Nigeria

3

Mechanical Engineering Department, University of Ibadan, Ibadan, Nigeria Email: temitayo.fatoki@covenantuniversity.edu.ng

Received 25 February 2015; accepted 22 June 2015; published 25 June 2015

Copyright © 2015 by authors and Scientific Research Publishing Inc.

This work is licensed under the Creative Commons Attribution International License (CC BY). http://creativecommons.org/licenses/by/4.0/

# Abstract

The dispersion models were used to estimate or predict the concentration of air pollutants or toxins emitted from sources such as industrial plants, vehicular traffic or accidental chemical releases. In this study, the Industrial Source Complex Short Term (ISCST3) emission dispersion model was used to measure the ground level concentration of criteria air pollutants within 50 km radius of location. This model considered emissions from major point sources of pollutants in four existing and twenty-three proposed Nigeria petroleum refineries. The obtained ground level concentration for 24-hr averaging periods of the criteria air pollutants at sensitive receptor around each of the refineries was compared with the National Ambient Air Quality Standards (NAAQS) of Nigeria, World Bank and World Health Organization (WHO) to determine their level of compliance. The highest ground level concentration predicted to be 450 - 1875 µg/m3 for 24-h averaging period was obtained at Tonwei Oil Refinery, Ekeremor Local Government, Bayelsa State, while the lowest ground level concentration predicted to be 0.0099 - 0.1 µg/m3 for 24-h averaging period was obtained at Amakpe International Refinery, Eket Local Government, Akwa Ibom State. Percentage set limits of criteria air pollutants ranging from 0.02% to 94.5% are within the set standard limits and no health risk is associated with areas around the plant’s locations while percentage set limits of criteria pollutants ranging from 1.1 to 55.6 folds of the standards exceed the maximum permitted limits, hence affecting areas around the plants. The air quality standards guiding petroleum refinery emissions must be strictly considered, in order to ensure that the ground level concentrations do not exceed the required standard limits and prevent the adverse effects of air pollution in the Nigeria airshed.

# Keywords

Air Quality Standards, Ground Level Concentration, Nigeria Petroleum Refinery

How to cite this paper: Oladimeji, T., Sonibare, J., Odunfa, M. and Ayeni, A. (2015) Modeling of Criteria Air Pollutant Emissions from Selected Nigeria Petroleum Refineries. Journal of Power and Energy Engineering, 3, 31-45. http://dx.doi.org/10.4236/jpee.2015.36004

# Introduction

Each year industrial facilities discharge pollutants into the environment, releasing large amounts of pollutants leading to respiratory, neurological and reproductive disorders, and cancers [1]. Nigeria is presently a major emitter of criteria air emissions: carbon monoxide (CO), oxides of nitrogen (NOx), particulate matter less than 10.0 microns in diameter (PM10), sulphur dioxide (SO2) and volatile organic compounds (VOCs) which comes from number of sources within a petroleum refinery including equipment leaks (from valves or other devices), high- temperature combustion processes, etc [2]. The Industrial Source Complex Short Term (ISCST3) air dispersion modeling tool is a steady state Gaussian plume model which can be used to assess pollutant concentrations, and/ or deposition fluxes from a wide variety of sources are associated with an industrial source complex [3]. It was used to predict the ground level concentrations of criteria air pollutants from the four major existing and twenty-three proposed petroleum refineries in Nigeria, within 50 km radius of location from point sources of emission in the refineries, for the prevailing meteorological conditions assuming a “worst case scenario” i.e. 100% operation rate. The model is used to determine whether existing or proposed new industrial facilities are or will be in compliance with the National Ambient Air Quality Standards (NAAQS). Air pollution models constitute a set of formulae that take into account sources of pollution in a given area, the amounts of pollutants emitted by each source, chemical reaction transformations, different meteorological conditions and topographical features, and other factors that affect dispersion of pollutants. [4] defined the process of computer modeling of air pollution as the ability to use computers to solve basic equations that described the dynamics of meteorological and air pollution phenomena, and ultimately, the adverse effects of air pollution. The basis for most of those models was the complete equation for Gaussian Dispersion Modeling of continuous and buoyant air pollution plumes [5]. Previous studies on investigation on emissions using ISCST3 include [5]-[7]. This study is carried out to investigate the atmospheric conditions of the petroleum refineries in Nigeria and identify the environmental impacts of emissions of criteria pollutants in the area of influence. Determinations of their ground level concentrations and comparison with the National Ambient Air Quality Standards (NAAQS) of Nigeria, World Bank and World Health Organization (WHO) will indicate the level of air quality management required around Nigerian petroleum refineries.

# Study Areas

As shown in Figure 1, Nigeria has 4 existing refineries; Kaduna Refining and petrochemical company (KPRC) located at Chikun Local Government of Kaduna State operating at 110,000 bbl/day; Warri Refining and petrochemical company (WRPC) located at Warri southwest local government of Nigeria’s Delta State with an installed capacity of 125,000 bbl/day; Port Harcourt Refining Company (PHRC) has two refineries at Alesa-Eleme, near Port Harcourt in Nigeria’s Southern Province, Rivers State. Port Harcourt I refinery has installed capacity of 60,000 bbl/day while Port Harcourt II refinery is the complex new refinery with installed capacity of 150,000 bbl/day [8] [9]. Due to scarcity of petroleum products, 23 proposed refineries have been licensed in Nigeria to ensure availability of the products (Figure 1), which includes the following with their installed capacities; Resources refinery and petrochemical Limited located in Ikot Abasi local government area of Akwa Ibom state (100,000 bbl/day); Amakpe International Refinery, Eket local government of Akwa Ibom state with production capacity of (12,000 bbl/day); Rehoboth Natural Resources Limited (12,000 bbl/day), Immingiri, Yenekoa local government of Bayelsa state; Amexum Corporation (100,000 bbl/day), Akpabuyo local government of Cross river state; Antonio Oil (27,000 bbl/day), Ipokia local government of Ogun state; Gasoline Associates International Limited Refinery (100,000 bbl/day), Ipokia local government of Ogun state; Ologbo Refinery Company Nigeria Limited (12,000 bbl/day), Ikpoba Okha local government of Edo state; Ode Aye Refinery Limted (100,000 bbl/day), Okiti pupa local government area of Ondo state; Orient Petroleum Resources Limited (55,000 bbl/day), Anambra West local government area of Anambra state; Qua Petroleum Refinery Limited (100,000 bbl/day), Ibeno local government of Akwa Ibom state; ChaseWood Consortium Nigeria Limited (70,000 bbl/day), Eket local government area of Akwa Ibom state; Clean Water Refinery (60,000 bbl/day), Eleme local government area of Rivers state; Niger Delta Refinery & Petrochemical Limited (100,000 bbl/day), Warri-South local government area of Delta state; NSP Refineries and Oil Services Limited (120,000 bbl/day), Andoni local government area of Rivers state; RIVGAS Petroleum and Energy Limited (30,000 bbl/day), Port Harcourt local government area of Rivers state; Sapele Refinery Limited located in Okpe-Sobo, Sapele local government area of Delta state (100,000 bbl/day); Starrex Petroleum Refinery (100,000 bbl/day), Eleme local government area of

Figure 1. Map of Nigeria showing the existing and proposed Refineries.

Rivers state; Total Support Limited (12,000 bbl/day), Calabar Mnincipal local government area of Cross River state; Union Atlantic Petroleum Limited (100,000 bbl/day), Badagry local government area of Lagos state; Tonwei Oil Refinery (200,000 bbl/day), Ekeremor local government area of Bayelsa state; Ilaje Refinery and Petrochemicals (100,000 bbl/day), Ilaje local government area of Ondo state; Southwest Refinery and Petrochemical Company (100,000 bbl/day) is located in Ogun Waterside local government area of Ogun state; Owena Oil and gas Limited (100,000 bbl/day), Ilaje local government area of Ondo state [8]-[10].

# Methodology

ISCST3 model uses steady state Gaussian plume equation to model emissions from point sources, including flares, process heaters, steam boilers and fluid catalytic cracking unit. This model considered emissions from the stacks of existing refineries and proposed petroleum refineries in Nigeria. Criteria air pollutants modeled for the ground level concentration include CO, NOX, PM10, SO2 and VOC. The standards of ambient air quality as reported by NAAQS,WB and WHO is shown in Table 1. Major sources of emissions and parameters used as input to the air dispersion modeling for each of the refineries are shown in Table 2 and Table 3. Some proposed refineries have similar operation rate, hence will have the same emission sources and dispersion model parameter. The ISC-AERMOD View air dispersion modeling tool developed specially for Microsoft Windows and runs under Windows 95/98/Me/NT/2000 and XP was employed in this study. This is a complete and powerful air dispersion modeling package which incorporates the popular U.S. EPA models into one interface: AERMOD, ISCST3, and ISC-PRIME. ISCST3 dispersion model interfaces [1]. The ISCST3 interface was used in this study which uses pathways that compose the run stream file as the basics for its functional organization. These pathways include:

i) Control Pathway (CO): Where the modeling scenario (worst case scenario i.e. 100% operation rate), the

pollutants (CO, NOX, SO2, VOC and PM) and the averaging periods (24-hr) are specified. ii) Source Pathway (SO): Where the sources of pollutant emissions (point sources of pollutants in the refinery-process heaters, fluid catalytic cracking units, steam boilers and flares) are defined.

Table 1. Standards of ambient air quality.

| S/N | Pollutants | Averaging Period | Maximum Concentration (µg/m3) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | NAAQS | World Bank | WHO |
| 1 | CO | 24-Hr | 11,400 | - | - |
| 2 | NOx | 24-Hr | 75 | 150 | - |
| 3 | SO2 | 24-Hr | 26 | - | 20 |
| 4 | PM | 24-Hr | 250 | 80 | 50 |
| 5 | VOC | 24-Hr | 6000 | - | - |

Source: FEPA (1991) [11]; World Bank (1998) [12]; WHO (2005) [13].

Table 2. Emission sources and dispersion model parameters for existing refineries.

| Plant Name | Stack | Discharge Temperature (k) | Stack Diameter (m) | Exit Velocity (m/s) | Release Height (m) |  |  | c Emission (g/s) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | PM10 | SO2 | NOX | VOC | CO |
| Kaduna Refining & Petrochemical Company (KRPC) 110,000 BPSD | a Flare 1 Flare 2 Flare 3 b FCCU b Boiler 1 Boiler 2 | 1540 1540 1540 330 550 550 | 6.6 6.6 6.6 2.75 2.18 2.18 | 4.52 4.52 4.52 23.47 9.05 9.05 | 66 66 66 67 30 30 | - - - - - - | - - - 54.5 304.1 304.1 | 0.001 0.001 0.001 7.9 1420.1 1420.1 | 0.003 0.003 0.003 24.3 - - | 0.005 0.005 0.005 1510.4 106 106 |
|  | Boiler 3 | 550 | 2.18 | 9.05 | 30 | - | 304.1 | 1420.1 | - | 106 |
|  | b Heater 1 | 660 | 2.01 | 8.03 | 43 | 2.1 | 45.1 | 173.2 | 1.0 | 15.8 |
|  | Heater 2 | 660 | 2.01 | 8.03 | 43 | 2.1 | 45.1 | 173.2 | 1.0 | 15.8 |
| Port Harcourt Refining Company I (PHRC) 60,000 BPSD | a Flare 1 Flare 2 b FCCU b Boiler 1 Boiler 2 | 840 840 180 300 300 | 1.1 1.1 1.5 1.19 1.19 | 2.47 2.47 12.8 4.94 4.94 | 36 36 37 16 16 | - - - - - | - - 12.5 59.6 59.6 | 0.0003 0.0003 2.0 278.7 278.7 | 0.0006 0.0006 5.6 - - | 0.002 0.002 347.7 20.8 20.8 |
|  | b Heater 1 | 360 | 1.1 | 4.38 | 24 | 0.6 | 13.4 | 51.6 | 0.3 | 4.7 |
|  | Heater 2 | 360 | 1.1 | 4.38 | 24 | 0.6 | 13.4 | 51.6 | 0.3 | 4.7 |
| Port Harcourt Refining Company II (PHRC) 150,000 BPSD | a Flare 1 Flare 2 Flare 3 Flare 4 b FCCU b Boiler 1 Boiler 2 | 2100 2100 2100 2100 450 750 750 | 2.75 2.75 2.75 2.75 3.75 2.97 2.97 | 6.17 6.17 6.17 6.17 32.01 12.35 12.35 | 90 90 90 90 92 41 41 | - - - - - - | - - - 103.5 267.8 267.8 | 0.002 0.002 0.002 0.002 14.9 1250.7 1250.7 | 0.004 0.004 0.004 0.004 46.3 - - | 0.01 0.01 0.01 0.01 2877 93.4 93.4 |
|  | b Boiler 1 | 750 | 2.97 | 12.35 | 41 | - | 267.8 | 1250.7 | - | 93.4 |
|  | Boiler 2 | 750 | 2.97 | 12.35 | 41 | - | 267.8 | 1250.7 | - | 93.4 |
|  | b Heater 1 | 900 | 2.75 | 10.95 | 59 | 3.7 | 80.2 | 308.1 | 1.7 | 28 |
|  | Heater 2 | 900 | 2.75 | 10.95 | 59 | 3.7 | 80.2 | 308.1 | 1.7 | 28 |

## Continued

| Warri Refining & Petrochemical Company (WRPC) 125,000 BPSD | a Flare 1 Flare 2 Flare 3 Flare 4 b FCCU b Boiler 1 Boiler 2 | 1750 1750 1750 1750 375 625 625 | 2.29 2.29 2.29 2.29 3.13 2.48 2.48 | 5.13 5.13 5.13 5.13 26.68 10.29 10.29 | 75 75 75 75 76 34 34 | - - - - - - | - - - 67.3 228.8 228.8 | 0.001 0.001 0.001 0.001 9.7 1068.3 1068.3 | 0.003 0.003 0.003 0.003 30 - - | 0.007 0.007 0.007 0.007 1870.1 79.8 79.8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Boiler 3 | 625 | 2.48 | 10.29 | 34 |  | 228.8 | 1068.3 | - | 79.8 |
|  | b Heater 1 | 750 | 2.29 | 9.13 | 49 | 2.9 | 61.3 | 253.3 | 1.3 | 21.4 |
|  | Heater 2 | 750 | 2.29 | 9.13 | 49 | 2.9 | 61.3 | 253.3 | 1.3 | 21.4 |

Source: aAbdul-Wahab et al. (2001) [14]; bStell (2000) [15]; cUSEPA 1989 [16].

Table 3. Emission sources and dispersion model parameters for proposed refineries.

| Plant Name | Stack | Discharge Temperature (k) | Stack Diameter (m) | Exit Velocity (m/s) | Release Height (m) |  |  | c Emission (g/s) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | PM10 | SO2 | NOX | VOC | CO |
| Resource Refinery & Petrochemical, Onne 100,000BPSD | a Flare 1 Flare 2 Flare 3 b FCCU b Boiler 1 Boiler 2 | 1400 1400 1400 300 500 500 | 1.83 1.83 1.83 2.5 1.98 1.98 | 4.11 4.11 4.11 21.34 8.23 8.23 | 60 60 60 61 27 27 | - - - - - - | - - - 34.7 165.7 165.7 | 0.001 0.001 0.001 5.0 773.9 773.9 | 0.002 0.002 0.002 15.6 - - | 0.004 0.004 0.004 965.9 57.8 57.8 |
|  | b Heater 1 | 600 | 1.83 | 7.3 | 39 | 1.7 | 37.3 | 143.2 | 0.8 | 13 |
|  | Heater 2 | 600 | 1.83 | 7.3 | 39 | 1.7 | 37.3 | 143.2 | 0.8 | 13 |
| NSP Refineries & Oil services Limited, Andoni 120,000BPSD | a Flare 1 Flare 2 Flare 3 Flare 4 b FCCU b Boiler 1 | 1680 1680 1680 1680 360 600 | 2.2 2.2 2.2 2.2 3.0 2.38 | 4.93 4.93 4.93 4.93 25.61 10.0 | 72 72 72 72 73 33 | - - - - - | - - - 50.1 238.6 | 0.001 0.001 0.001 0.001 7.1 1114.4 | 0.003 0.003 0.003 0.003 22.3 - | 0.006 0.006 0.006 0.006 1390.9 83.2 |
|  | Boiler 2 | 600 | 2.38 | 10.0 | 33 | - | 238.6 | 1114.4 | - | 83.2 |
|  | b Heater 1 | 720 | 2.2 | 8.76 | 47 | 2.5 | 53.7 | 206.1 | 1.1 | 18.7 |
|  | Heater 2 | 720 | 2.2 | 8.76 | 47 | 2.5 | 53.7 | 206.1 | 1.1 | 18.7 |
| Clean Water Refinery, Eleme (60,000BPSD | a Flare 1 Flare 2 b FCCU b Boiler 1 | 840 840 180 300 | 1.1 1.1 1.5 1.19 | 2.47 2.47 12.8 4.94 | 36 36 37 16 | - - - - | - - 12.5 59.6 | 0.0003 0.0003 2.0 278.7 | 0.0006 0.0006 5.6 - | 0.002 0.002 347.7 20.8 |
|  | Boiler 2 | 300 | 1.19 | 4.94 | 16 | - | 59.6 | 278.7 | - | 20.8 |
|  | b Heater 1 | 360 | 1.1 | 4.38 | 24 | 0.6 | 13.4 | 51.6 | 0.3 | 4.7 |
|  | Heater 2 | 360 | 1.1 | 4.38 | 24 | 0.6 | 13.4 | 51.6 | 0.3 | 4.7 |

## Continued

| RIVGAS Petroleum & Energy Limited, Port Harcourt (30,000BPSD) | a Flare 1 Flare 2 b FCCU b Boiler 1 Boiler 2 | 420 420 90 150 150 | 0.55 0.55 0.75 0.59 0.59 | 1.23 1.23 6.4 2.47 2.47 | 18 18 18 8 8 | - - - - - | - - 3.1 14.9 14.9 | 0.00007 0.00007 0.4 69.6 69.6 | 0.00002 0.00002 1.4 - - | 0.0004 0.0004 87 5.2 5.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | b Heater 1 | 180 | 0.55 | 2.19 | 12 | 0.2 | 3.4 | 12.9 | 0.07 | 1.2 |
|  | Heater 2 | 180 | 0.55 | 2.19 | 12 | 0.2 | 3.4 | 12.9 | 0.07 | 1.2 |
| Towei Oil Refinery 200,000 BPSD | a Flare 1 Flare 2 Flare 3 Flare 4 b FCCU b Boiler 1 | 2800 2800 2800 2800 600 1000 | 3.66 3.66 3.66 3.66 5.0 3.96 | 8.22 8.22 8.22 8.22 42.68 16.46 | 120 120 120 120 122 54.3 | - - - - - - | - - - 2158.7 23,040 | 0.11 0.11 0.11 0.11 312.85 107605.4 | 0.20 0.20 0.20 0.20 962.9 - | 0.70 0.70 0.70 0.70 60,006 8,033.4 |
|  | Boiler 2 | 1000 | 3.96 | 16.46 | 54.3 | - | 23,040 | 107605.4 | - | 8,033.4 |
|  | Boiler 3 | 1000 | 3.96 | 16.46 | 54.3 | - | 23,040 | 107605.4 | - | 8,033.4 |
|  | Boiler 4 | 1000 | 3.96 | 16.46 | 54.3 | - | 23,040 | 107605.4 | - | 8,033.4 |
|  | b Heater 1 | 1200 | 3.66 | 14.6 | 78.6 | 229.86 | 5183.0 | 19904.66 | 107.7 | 1,811.1 |
|  | Heater 2 | 1200 | 3.66 | 14.6 | 78.6 | 229.86 | 5183.0 | 19904.66 | 107.7 | 1,811.1 |
| Orient Petroleum Resource Limited 55,000 BPSD | a Flare 1 Flare 2 b FCCU b Boiler 1 Boiler 2 | 770 770 165 275 275 | 1.01 1.01 1.38 1.09 1.09 | 2.26 2.26 11.74 4.53 4.53 | 33 33 34 15 15 | - - - - - | - - 368.48 1741.5 1741.5 | 0.001 0.001 55.62 8137.76 8137.76 | 0.03 0.03 163.3 - - | 0.04 0.04 10,167 608.33 608.33 |
|  | b Heater 1 | 330 | 1.01 | 4.02 | 22 | 17.38 | 392.81 | 1505.19 | 10.43 | 139.05 |
|  | Heater 2 | 330 | 1.01 | 4.02 | 22 | 17.38 | 392.81 | 1505.19 | 10.43 | 139.05 |
| Amakpe Internatna Refinery 12,000 BPSD | a Flare 1 Flare 2 b FCCU b Boiler 1 Boiler 2 | 168 168 36 60 60 | 0.22 0.22 0.3 0.24 0.24 | 0.49 0.49 2.56 0.98 0.98 | 7 7 7 3 3 |  | - - 13.70 560.05 560.05 | 0.0004 0.0004 2.43 2602.24 2602.24 | 0.001 0.001 10.43 - - | 0.003 0.003 483.19 209.24 209.24 |
|  | b Heater 1 | 72 | 0.22 | 0.88 | 5 | 1.05 | 1.05 | 73.00 | 0.35 | 0.69 |
|  | Heater 2 | 72 | 0.22 | 0.88 | 5 | 1.05 | 1.05 | 73.00 | 0.35 | 0.69 |
| Chase wood Consortiu Nigeria Limited 70,000 BPSD | a Flare 1 Flare 2 b FCCU b Boiler 1 Boiler 2 | 980 980 210 19 19 | 1.28 1.28 1.75 350 350 | 2.88 2.88 14.94 1.39 1.39 | 42 42 43 19 19 | - - - - - | - - 590.95 2930.4 2930.4 | 0.01 0.01 83.42 13678.8 13678.8 | 0.03 0.03 264.1 - - | 0.07 0.07 13.90 983.8 983.8 |
|  | b Heater 1 | 420 | 1.28 | 5.11 | 28 | 31.28 | 636.14 | 2436.81 | 13.90 | 222.48 |
|  | Heater 2 | 420 | 1.28 | 5.11 | 28 | 31.28 | 636.14 | 2436.81 | 13.90 | 222.48 |

Source: aAbdul-Wahab et al. (2001); bStell (2000); cUSEPA (1989).

- Receptor Pathway (RE): where the receptors which determine the air quality impact at specific locations. Specifically, 50 km radius within each refinery was given adequate attention.

- Meteorological Pathway (ME): Where the atmospheric condition of the areas being modeled is defined, so it can be taken into account when determining the distribution of air pollution impacts for the area. Due to lack of upper air observations in Nigeria airport, meteorological database for the project location was combined using data from Cotonou meteorological observations. Cotonou has winds having prevalence for a southwesterly direction which is consistent with the winds observed in the refineries.

- Terrain Grid Pathway (TG): Where the option of gridded terrain data to be used in calculating dry deple-

tion in elevated or complex terrain is specified. Elevated terrain is relevant to this study.

(vi) Output Pathway (OU): Where you define which output results are necessary to meet the needs of the air quality modeling analysis. The resultant output is the ground level concentration of pollutants as shown in Figure 2 and Figure 3.

(a)                                                            (b)

(c)                                                            (d)

(e)

Figure 2. Predicted 24-hr Ground Level Concentrations of Criteria Pollutants from Kaduna Refining and Petrochemical

Company, Kaduna State. (a) Predicted 24-hr PM10 Concentrations; (b) Predicted 24-hr VOC Concentrations; (c) Predicted

24-hr SO2 Concentrations; (d) Predicted 24-hr NOX Concentrations; (e) Predicted 24-hr CO Concentrations.

(a)                                                           (b)

(c)                                                            (d)

(e)

Figure 3. Cumulative 24-hr Ground Level Concentrations of Criteria Pollutants from Bayelsa state refineries. (a): Cumula-

tive 24-hr PM10 Concentrations; (b) Cumulative 24-hr SO2 Concentrations; (c) Cumulative 24-hr NOX Concentrations; (d) Cu-

mulative 24-hr VOC Concentrations; (e) Cumulative 24-hr SO2 Concentrations.

From the existing and proposed refineries, the ground level concentration (glc) of the criteria air pollutant were computed and compared with the standards of ambient air quality derived from National Ambient Air Quality Standards (NAAQS) of Federal Ministry of Environment (FMENV), that of the World Bank environmental guidelines and World Health Organization (WHO) standards to determine their level of compliance (Table 4 and Table 5).

# Results and Discussion

An air quality impact on refineries is important to assess the impacts of emissions from petroleum refinery operations on their environment. Table 2 and Table 3 shows the emission sources and dispersion model parameters for the existing and proposed refineries these include the emission estimate of criteria pollutants from point sources (FCCU, Process heaters, Flares, Steam boilers) of emissions from the refineries. The dispersion modeling outputs from the ISC-AERMOD View (for the existing and proposed refineries in Nigeria) for 1-hour, 8- hour, 24-hour and annual averaging periods for the criteria pollutants are shown in Figure 2 and Figure 3

| Table 4. 24-hr averaging period maximum ground level concentration of criteria pollutants and % set limits in existing refi- |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| neries. |  |  |  |  |  |  |
| S/N | Refinery | Air Pollutant | Maximum glc (µg/m3) | Maximum Impact Local Government Area | % Set Limit of FMENV, WB, WHO |
| 1 | Kaduna Refining & Petrochemical Company (KRPC) 110,000 BPSD | PM10 SO2 NOx CO | 0.4 82.7 404.9 164.1 | Chikun Chikun Chikun Chikun | 0.2 3.2a 5.4a 1.4 |
|  |  | VOC | 2..3 | Chikun | 0.04 |
| 2 | Port Harcourt Refining Company I (PHRC) 60,000 BPSD | PM10 SO2 NOx CO | 0.2 26.3 121.4 73.0 | Eleme Eleme Eleme Eleme | 0.08 1.02a 1.6a 0.6 |
|  |  | VOC | 1.0 | Eleme | 0.02 |
| 3 | Port Harcourt Refining Company II (PHRC) 150,000 BPS | PM10 SO2 NOx CO | 0.7 111.9 3,589.1 214.9 | Eleme Eleme Eleme Eleme | 0.3 4.3a 4.7a 1.9 |
|  |  | VOC | 17.6 | Eleme | 0.3 |
| 4 | Warri Refining & Petrochemical Company (WRPC) 125,000 BPSD | PM10 SO2 NOx CO | 1.7 444.4 2,145.8 479.2 | Warri S/W Warri S/W Warri S/W Warri S/W | 0.7 1.7a 2.9a 4.2 |
|  |  | VOC | 0.9 | Warri S/W | 0.1 |

a folds of % set limits of the standard i.e. above the set standard limits.

(surface plots available on request) while the Tables showing the results of maximum ground Level Concentration of air pollutant within 50 km from each of the refineries are shown in Table 4 and the maximum ground level concentration impact from 50 km receptor locations obtained when the ground level concentration of the criteria pollutants from the refineries was compared with the set limits of Nigeria’s National Ambient Air Quality Standard (NAAQS), World Bank and World Health Organization (WHO) are shown in Table 5 and Table 6. The results predicted the total emissions of criteria pollutant emitted per annum in the existing and proposed refineries where NOx has the highest rate of emissions. Also, from the existing refineries, Port Harcourt 2 refinery has the highest criteria pollutant emission rate due to its high installed capacity (150,000 bpsd) while from the proposed refineries Tonwei refinery has the highest rate of emissions of the criteria pollutant due to its high installed capacity (200,000 bpsd). It was observed that some refineries exceeds their area of influence i.e. the emission dispersion exceeds the plant location to the neighbouring local government area e.g. CO emissions for Rehoboth for 1 h averaging periods dispersed from Yenegoa to Ogbia. From Table 5 and Table 6, criteria pollutants of ranges 0.02% - 94.5% of the standards falls within the set standards limit which means there is no health risk in the receptor locations while criteria pollutants of ranges 1.1 - 55.6 folds of the standards have exceeded the set standard limits and it means there is health risk in the receptor locations.

# Conclusions

The ISCST3 dispersion model has been used in this study to evaluate the ground level concentrations from the

| Table 5. 24-hr averaging period maximum ground level concentration of criteria pollutants and % set limits in proposed re- |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| fineries. |  |  |  |  |  |  |
| S/N | Refinery | Air Pollutant | Maximum glc (µg/m3) | Maximum Impact Local Government Area | % Set Limit of FMENV, WB, WHO |
| 1 | Amakpe International Refinery (12,000 bpsd) | PM10 SO2 NOx | 0.04 4.4 22.6 | Ikot Ekpene Ikot Ekpene Ikot Ekpene | 0.02 16.9 30.1 |
|  |  | CO | 10.7 | Ikot Ekpene | 0.04 |
|  |  | VOC | 0.2 | Ikot Ekpene | 0.002 |
| 2 | Amexum Corporation (100,000 bpsd) | PM10 SO2 NOx | 0.5 6.7 314 | Akpabuyo Akpabuyo Akpabuyo | 0.2 0.4 0.4a |
|  |  | CO | 215 | Akpabuyo | 1.9 |
|  |  | VOC | 4.1 | Akpabuyo | 0.1 |
| 3 | Antonio Oil (27,000 bpsd) | PM10 SO2 NOx | 0.5 99.4 413 | Ipokia Ipokia Ipokia | 0.2 3.8a 1.5 |
|  |  | CO | 37.5 | Ipokia | 0.3 |
|  |  | VOC | 1.8 | Ipokia | 0.03 |
| 4 | Chase Wood Consortium Nigeria Limited (70,000 bpsd) | PM10 SO2 NOx | 0.5 210 1500 | Eket Eket Eket | 0.2 0.8a 2.0a |
|  |  | CO | 170 | Eket | 0.01 |
|  |  | VOC | 26 | Eket | 0.4 |
| 5 | Clean Water Refinery (60,000 bpsd) | PM10 SO2 NOx | 0.5 39.5 185.5 | Eleme Eleme Eleme | 0.2 1.3a 2.0a |
|  |  | CO | 102.4 | Eleme | 0.9 |
|  |  | VOC | 1.6 | Eleme | 0.02 |
| 6 | Gasoline Associate & International Limited Refinery (100,000 bpsd | PM10 SO2 NOx CO | 0.5 58.6 281.5 139.8 | Ipokia Ipokia Ipokia Ipokia | 0.2 2.3a 3.8a 1.2 |
|  |  | VOC | 2.1 | Ipokia | 0.04 |
| 7 | Ilaje Refinery & Petrochemicals (100,000 bpsd) | PM10 SO2 NOx | 0.8 61 298 | Ilaje Ilaje Ilaje | 0.3 0.2a 3.9a |
|  |  | CO | 192 | Ilaje | 1,7 |
|  |  | VOC | 3.0 | Ilaje | 0.05 |

0.5 	Warri South 	0.2

72.7 	Warri South 	2.8a

Niger Delta Refinery &

- Petrochemical Limited 	347.3 	Warri South 	4.6a

(100,000 bpsd)

417.1 	Warri South 	3.7

VOC 	6.7 	Warri South 	0.1

PM10 	0.9 	Andoni 	0.4

SO2 	395.0 	Andoni 	1.5a

NSP Refineries and Oil

- Services Limited  	NOx 	1903.0 	Andoni 	1.3a

(120,000 bpsd)

CO 	375.0 	Andoni 	3.3

VOC 	6.0 	Andoni 	0.1

| 10 | Ode Aye Refinery Limited (100,000 bpsd) | PM10 SO2 NOx CO | 0.4 83.5 440 113.8 | Okiti pupa Okiti pupa Okiti pupa Okiti pupa | 0.2 32.1a 5.9a 1.0 |
| --- | --- | --- | --- | --- | --- |
|  |  | VOC | 2.0 | Okiti pupa | 0.03 |
| 11 | Ologbo Refinery Company Nigeria Limited (12,000 bpsd) | PM10 SO2 NOx | 0.05 0.9 8.9 | Ikpoba-oha Ikpoba-oha Ikpoba-oha | 0.02 3.5a 11.8 |
|  |  | CO | 5.0 | Ikpoba-oha | 0.04 |
|  |  | VOC | 0.1 | Ikpoba-oha | 0.002 |
| 12 | Orient Petroleum Resources Limited (55,000 bpsd) | PM10 SO2 NOx | 0.7 0.9 8.5 | Anambra West Anambra West Anambra West | 0.3 3.5 11.3 |
|  |  | CO | 219.6 | Anambra West | 1.9 |
|  |  | VOC | 3.0 | Anambra West | 0.01 |
| 13 | Owena Oil & Gas Limited (60,000 bpsd) | PM10 SO2 NOx | 0.2 34.5 159.4 | Ilaje Ilaje Ilaje | 0.08 13.3a 2.2a |
|  |  | CO | 78 | Ilaje | 0.7 |
|  |  | VOC | 1.2 | Ilaje | 0.02 |
| 14 | Qua Petroleum Refinery Limited (100,000 bpsd) | PM10 SO2 NOx | 1.0 142.5 623 | Ibeno Ibeno Ibeno | 0.4 24.0a 8.3a |
|  |  | CO | 423 | Ibeno | 3.7 |
|  |  | VOC | 6.5 | Ibeno | 0.1 |

0.05 	Yenegoa 	0.02

2.4 	Yenegoa 	92.3

Rehoboth Natural

- Resources Limited 	12.1 	Yenegoa 	16.1

(12,000 bpsd

9.7 	Yenegoa 	0.1

VOC 	0.2 	Yenegoa 	0.003

PM10 	1.3 	Ikot Abasi 	0.7

SO2 	191.2 	Ikot Abasi 	8.6a

Resources Refinery &

- Petrochemical Limited 	NOx 	897.3 	Ikot Abasi 	14.8a

(100,000 bpsd)

CO 	399.4 	Ikot Abasi 	5.8

| 19 | South West Refinery & Petrochemical Company (100,000 bpsd) | PM10 SO2 NOx CO | 0.7 59.4 270 149.8 | Ogun water side Ogun water side Ogun water side Ogun water side | 0.3 22.8a 3.6a 1.3 |
| --- | --- | --- | --- | --- | --- |
|  |  | VOC | 2.4 | Ogun water side | 0.04 |
| 20 | Starrex Petroleum Refinery (100,000 bpsd) | PM10 SO2 NOx | 1.0 0.4 112.3 | Eleme Eleme Eleme | 0.4 1.5 1.5a |
|  |  | CO | 3.9 | Eleme | 0.03 |
|  |  | VOC | 1.8 | Eleme | 0.03 |
| 21 | Towei Oil Refinery (200,000 bpsd) | PM10 SO2 NOx | 1.5 376 1875.4 | Ekeremor Ekeremor Ekeremor | 0.6 1.0a 2.5a |
|  |  | CO | 168.9 | Ekeremor | 1.5 |
|  |  | VOC | 1.6 | Ekeremor | 0.03 |

VOC 	6.0 	Ikot Abasi 	0.2

PM10 	0.3 	Port Harcourt 	0.1

SO2 	0.4 	Port Harcourt 	1.5

RIVGAS Petroleum &

- Energy Limited 	NOx 	112.3 	Port Harcourt 	1.5a

(30,000 bpsd)

CO 	3.9 	Port Harcourt 	0.03

VOC 	1.8 	Port Harcourt 	0.02

PM10 	0.3 	Sapele 	0.1

SO2 	48.9 	Sapele 	18.8a

Sapele Refinery Limited

- NOx 	255.2 	Sapele 	3.4a

(100,000 bpsd)

CO 	48.9 	Sapele 	0.4

VOC 	1.6 	Sapele 	0.03

| 22 | Total Support Limited (12,000 bpsd) |  | 0.7 2.9 16 | Calabar Municipal Calabar Municipal Calabar Municipal | 0.3 6.2a 21.3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 22 | Calabar Municipal | 0.2 |
|  |  | VOC | 0.5 | Calabar Municipal | 0.01 |
| 23 | Union Atlantic Petroleum Refinery (100,000 bpsd) | PM10 SO2 NOx | 1.8 224 116.8 | Badagry Badagry Badagry | 0.7 86.2a 1.6a |
|  |  | CO | 658 | Badagry | 5.8 |
|  |  | VOC | 10.3 | Badagry | 0.2 |

a folds of % set limits of the standard (above the set standard limit).

Table 6. Cumulative ground level concentration impact on ambient quality of multiple refineries within each state.

| S/N | Refinery | Air Pollutant | Cumulative Maximum glc (µg/m3) | % Set Limit of FMENV, WB, WHO |
| --- | --- | --- | --- | --- |
| 1 | Rehoboth Natural Resources Limited & Towei Oil Refinery Bayelsa State | PM10 SO2 NOx CO | 1.5 376 1875.4 168.9 | 0.6 14.5a 25.0a 1.5 |
|  |  | VOC | 1.6 | 0.03 |
| 2 | Total Support Limited & Amexum Corporation Cross River State | PM10 SO2 NOx | 0.5 7.0 314.4 | 0.2 2.7a 4.2a |
|  |  | CO | 215 | 1.9 |
|  |  | VOC | 2.3 | 0.1 |
| 3 | Antonio Oil, South West Refinery & Gasoline International Limited Refinery Ogun State | PM10 SO2 NOx CO | 4.6 107.4 426.4 149.8 | 1.8 4.1a 5.7a 1.7a |
|  |  | VOC | 2.6 | 0.04 |
| 4 | Ode Aye Refinery Limited, Ilaje Refinery & Petrochemicals & Owena Oil & Gas Limited Ondo state | PM10 SO2 NOx | 0.8 84.5 444.5 | 0.3 3.3a 5.9a |
|  |  | CO | 199.5 | 1.8 |
|  |  | VOC | 3.0 | 0.05 |
| 5 | Warri Refining, Niger Delta Refinery & Sapele Refinery Limited Delta State | PM10 SO2 NOx | 1.7 443.3 2145.8 | 0.7 17.1a 28.6a |
|  |  | CO | 480.3 | 4.2 |
|  |  | VOC | 7.0 | 0.1 |

| 6 | Amakpe International Refinery, Chase Wood Consortium Nigeria Limited, Resources Refinery & Qua Petroleum Refinery Limited Akwa Ibom State | PM10 SO2 NOx CO | 8.0 501 2214 480.3 | 3.2 19.3a 29.5a 3.7 |
| --- | --- | --- | --- | --- |
|  |  | VOC | 7.0 | 0.4 |
| 7 | Clean Water Refinery, NSP Refineries and Oil Services Limited, RIVGAS Petroleum & Energy Limited, Starrex Petroleum Refinery, Port Harcourt Refining Company I & Port Harcourt Refining Company II River state | PM10 SO2 NOx CO | 0.2 204 957 1.6 | 0.4 7.8a 12.8a 3.5 |
|  |  | VOC | 0.1 | 0.1 |

a folds of % set limits of the standard i.e. above the set standard limit.

four existing refineries and twenty-three proposed refineries in Nigeria for the purpose of predicting the air quality impact. Emissions from minimum of seven and maximum of eleven elevated point sources of criteria pollutants with “worst case” scenario were considered. The 24-hr averaging period of ground level concentration was measured and compared with the Nigeria’s ambient quality air set limits as set by National Ambient Air Quality Standard (NAAQS), World Bank and World Health Organization (WHO) within 50 km receptor locations.

From the modeling outputs, NOx and SO2 are the major pollutants that exceed the set limit, hereby causing harmful effects to the environment while PM, NOx and VOCs are within the set standard and have no effect on the environment. The highest ground level concentration predicted to be 450 - 1875 µg/m3 for 24-h averaging period was obtained at Tonwei Oil Refinery, Ekeremor Local Government, Bayelsa State, proposed to operate at installed capacity 200,000 barrels/day capacity while the lowest ground level concentration predicted to be 0.0099 - 0.1 µg/m3 for 24-h averaging period was obtained at Amakpe International Refinery, Eket Local Government, Akwa Ibom State, proposed to operate at installed capacity 12,000 barrels/day. According to Ambient Air Quality Standards (AAQS), percentage set limits of criteria air pollutants ranging from 0.02% - 94.5% are within the set standard limits and no health risk is associated with areas around the plant locations while percentage set limits of criteria pollutants ranging from 1.1 - 55.6 folds of the standards exceed the maximum permitted limits and health risk is associated with areas around the plant locations as stipulated by the National Ambient Air Quality Standard (NAAQS), World Bank and World Health Organization (WHO) emission set standards.

The study reveals that the contribution of emissions from criteria pollutant to its ambient air is significant. Possible impacts of anticipated emissions of criteria pollutants on humans [17], vegetations [18] soil and water [19] [20] are strong enough for necessary control measures to be implemented in both existing and proposed Nigerian petroleum refineries. Hence, the air quality standards guiding petroleum refinery emissions must be strictly considered, in order to ensure that the ground level concentrations do not exceed the required standard limits and prevent the adverse effect of air pollution in the Nigeria airshed.

# References

- [USEPA] United State Environmental Protection Agency (1999) Proper Monitoring Essential to Reducing Fugitive Emissions under Leak Detection and Repair Programs. Enforcement Alert, 2, 1-4.

- [WB] World Bank (1998) Petroleum Refining. Pollution Prevention and Abatement Handbook. World Bank Group, 3, 377-380.

- [WHO] World Health Organization (2005) WHO Air Quality Guidelines for Particulate Matter, Ozone, Nitrogen Dioxide and Sulphur Dioxide Global Update. Summary of Risk Assessment, 23, 1-20.

- Zannetti, P., Ed. (1994) Environmental Systems. Proceedings of ENVIROSOFT 94 Conference, Vol. II, San Francisco, November 1994, Computational Mechanics Publications, Southampton. http://www.witpress.com/

- Schnelle, K.B. and Dey, P.R. (1999) Atmospheric Dispersion Modeling Compliance Guide McGraw-Hill Professional. ISBN 0-07-058059-6.

- Elkamel, A., Abdul-Wahab, S., Bouhamra, W. and Alper, E. (2001) Measurement and Prediction of Ozone Levels around a Heavily Industrialized Area: A Neural Network Approach. Environmental Research, 5, 47-49. http://dx.doi.org/10.1016/S1093-0191(00)00042-3

- Sonibare, J.A. and Ede, P.N. (2009) Potential Impacts of Integrated Oil and Gas Plant on Ambient Air Quality. Energy & Environment, 20, 331-344. http://dx.doi.org/10.1260/095830509788066394

- [DPR] Department of Petroleum Resource (2001) Nigeria Oil Industry Statistical Bulletin. Department of Petroleum Resources, Ministry of Petroleum Resources, Lagos.

- [DPR] Department of Petroleum Resource (2004) Nigeria Oil Industry Statistical Bulletin. Department of Petroleum Resources, Ministry of Petroleum Resources, Lagos.

- [DPR] Department of Petroleum Resource (2010) Private Refineries and Petrochemical Plants Status. Nigeria Oil Industry Statistical Bulletin. Department of Petroleum Resources, Ministry of Petroleum Resources, Lagos.

- [FEPA] Federal Environmental Protection Agency (1991) National Interim Guideline and Standards for Industrial Effluents. Gaseous Emissions and Harzadous Wastes, 5, 33-63.

- [WB] World Bank (1998) Petroleum Refining. In: World Bank, Ed., Pollution Prevention and Abatement Handbook, 1998: Toward Cleaner Production, World Bank Group, Washington DC, 377-380.

- [WHO] World Health Organization (2005) WHO Air Quality Guidelines for Particulate Matter, Ozone, Nitrogen Dioxide and Sulphur Dioxide. Global Update 2005, Summary of Risk Assessment, 23, 1-14.

- Abdul-Wahab, S.A. and Al-Alawi, S. (2002) Assessment and Prediction of Tropospheric Ozone Concentration Levels Using Artificial Neural Networks. Environmental Modelling Software, 17, 219-228. http://dx.doi.org/10.1016/S1364-8152(01)00077-9

- Chang, T. and Stell, J. (2000) Worldwide Refining Survey. Oil Gas, 66-120.

- [USEPA] United States Environmental Protection Agency (1989) Airs Facility Subsystem Source Codes and Emission Factor Listing for Criteria Air Pollutants. Research Triangle Park, N.C. U.S. Environmental Protection Agency.

- Gou, H., Lee, S.C., Chan, L.Y. and Li, W.M. (2004) Risk Assessment of Exposure to Volatile Organic Compounds in

Different Indoor Environments. Environmental Research, 94, 57-66. http://dx.doi.org/10.1016/S0013-9351(03)00035-5

- Rao, P.S., Gavane, A.G., Ankam, P., Ansari, M.F., Pandit, V.I. and Nema, P. (2004) Performance Evaluation of a Green Belt in a Petroleum Refinery: A Case Study. Ecological Engineering, 23, 77-84. http://dx.doi.org/10.1016/j.ecoleng.2004.06.013

- Johnson, R.L., Thomas, T.B. and Zogorski, J.J. (2003) Effects of Daily Precipitation and Evapotranspiration Patterns on Flow and VOC Transport to Groundwater along a Watershed Flow Path. Environmental Science Technology, 37, 4944-4954. http://dx.doi.org/10.1021/es026252c

- Kim, H., Annabel, M.D. and Rao, P.S. (2001) Gaseous Transport of Volatile Organic Chemicals in Unsaturated Porous Media: Effect of Water Partitioning and Air-Water Interfacial Adsorption. Environmental Science Technology, 35, 4457-4462. http://dx.doi.org/10.1021/es001965l